Compression is a process intended to yield a compact digital representation of a signal.

source coding

data compression

bandwidth compression

image

video stream                          minimise the bit rate

audio signal

other

Without compression many applications would not be feasible!

Example 1: facsimile image transmission

The document is scanned and digitised.

Typically, an 8.5x11 inches page is scanned at 200 dpi; thus, resulting in 3.74 Mbits.

Transmitting this data over a low-cost 14.4 kbits/s modem would require 5.62 minutes.

With compression, the transmission time can be reduced to 17 seconds.

This results in substantial savings in transmission costs.

Example 2: video-based CD-ROM application

Full-motion video, at 30 fps and a 720 x 480 resolution, generates data at 20.736 Mbytes/s.

At this rate, only 31 seconds of video can be stored on a 650 MByte CD-ROM.

Compression technology can increase the storage capacity to 74 minutes, for VHS-grade video quality.

Why is compression possible?

Because there is considerable redundancy in the signal!

1.	Within a single image or a single video frame, there exists correlation among neighbour samples ⇒ spatial correlation

2.	For data acquired from multiple sensors (satellite images), there exists correlation amongst samples from these sensors

⇒ spectral correlation

3.	For temporal data (video), there is correlation amongst samples in different segments of time

⇒ temporal correlation

compression ratio cr used instead of bit rate to characterise the capability of the compression system

cr=sourcecoderinputsizesourcecoderoutputsize

This definition is somewhat ambiguous

depends on

data type

specific compression method

Example:	Still-image

size could refer to the bits needed to represent the entire image

Example:	Video

	size could refer to the bits needed to represent one frame of video

	size is the bits needed to represent one second of video

In the following figure, a systems view of the compression process is depicted.

Source Decoder

Channel

Decoder

DECODER

Source Coder

Channel

Coder

ENCODER

Digital image,

Video, Audio

Digital image,

Video, Audio

Figure: Generic compression system

| Application | Data | Rate |
| --- | --- | --- |
|  | Uncompressed | Compressed |
| Voice 8 ksamples/s, 8 bits/sample | 64 kbps | 2-4 kbps |
| Slow motion video (10fps) framesize 176x120, 8bits/pixel | 5.07 Mbps | 8-16 kbps |
| Audio conference 8 ksamples/s, 8 bits/sample | 64 kbps | 16-64 kbps |
| Video conference (15fps) framesize 352x240, 8bits/pixel | 30.41 Mbps | 64-768 kbps |
| Digital audio 44.1 ksamples/s, 16 bits/sample | 1.5 Mbps | 1.28-1.5 Mbps |
| Video file transfer (15fps) framesize 352x240, 8bits/pixel | 30.41 Mbps | 384 kbps |
| Digital video on CD-ROM (30fps) framesize 352x240, 8bits/pixel | 60.83 Mbps | 1.5-4 Mbps |
| Broadcast video (30fps) framesize 720x480, 8bits/pixel | 248.83 Mbps | 3-8 Mbps |
| HDTV (59.94 fps) framesize 1280x720, 8bits/pixel | 1.33 Gbps | 20 Mbps |

compression problem:	a bit rate minimisation problem with several constraints! as

	Specified level of signal quality. This constraint is usually applied at the decoder.

	Implementation complexity. This constraint is often applied at the decoder, and in some instances at both the encoder and the decoder.

	Communication delay. This constraint refers to the end to end delay, and is measured from the start of encoding a sample to the complete decoding of that sample.

Lossless compression

The reconstructed data and the original data must be identical in value for each and every data sample. This is also referred to as a reversible process.

Coding Efficiency                        Coding Delay

-Compression Ratio?

Coder Complexity

-Memory requirements?

-Power requirements?

-Operations per second?

Figure: Trade-offs in lossless compression.

Lossy compression

some amount of loss is permitted in the reconstructed data; irreversible process

Signal Quality

-Bit error probability?

-Signal/Noise?

-Mean opinion score?

Coding Efficiency                        Coding Delay

-Compression Ratio?

Coder Complexity

-Memory requirements?

-Power requirements?

-Operations per second?

Figure: Trade-offs in lossy compression.

Signal Quality

This term is often used to characterise the signal at the output of the decoder. There is no universally accepted measure for signal quality.

SNR

SNR=10log10encoderinputsignalenergynoisesignalenergy

noise = encoder input signal-decoder output signal

In the case of images or video, PSNR (peak signal-to-noise ratio) is used instead of SNR.

MEAN OPINION SCORE

The performance of a compression process is characterised by the subjective quality of the decoded signal.

For instance, a five point scale such as

very annoying

annoying

slightly annoying

perceptible but not annoying

imperceptible

might be used to characterise the impairments in the decoder output.

THE SOURCE CODER

original image data

1

decomposition

transformation

modelling

etc.

predictive coding

transform based coding

fractal coding

sub-band coding

Feature

selection

2

quantisation in finite number of levels

3

symbol encoding

could be Huffman

Compressed image

Figure: A generic representation

of a source coder

ELEMENTS OF INFORMATION THEORY

	Any information generating process can be viewed as a source that emits a sequence of symbols chosen from a finite alphabet.

Example: text: ASCII symbols

Example: n-bit images: 2n symbols.

	Simplest form of an information source: discrete memoryless source (DMS). Successive symbols produced by such a source are statistically independent.

A DMS is completely specified by the source alphabet S=s1,s2,…,sn and the associated probabilities p1,p2,…,pn.

	Self Information

I(si)=log21pi=−log2pi

- The occurrence of a less probable event provides more information.

- The information of independent events taken as a single event equals the sum of the information.

Example:

sk=sisj

pk=pipj

I(sk)=−log2pipj=−log2pi−log2pj=I(si)+I(sj)

	Average Information per Symbol or Entropy of a DMS

H(S)=i=1npiI(si)=−i=1npilog2pi bits/symbol

- Average amount of information per symbol provided by the source (definition).

- Average amount of information per symbol an observer needs to spend to remove the uncertainty in the source.

	N−th extention of the DMS

Given a DMS of size n, group the source into blocks of N symbols.

Each block can now be considered as a single source symbol generated by a source SN with alphabet size nN.

In this case it is proven that

H(SN)=N×H(s)

Noiseless Source Coding Theorem

Let S be a source with alphabet size n and entropy H(S).

Consider coding blocks of N source symbols into binary codewords.

For any δ>0, it is possible by choosing N large enough to construct a code in such a way that the average number of bits per original source symbol lavg satisfies

H(s)≤lavg≤H(s)+δ

The redundancy of a code is the difference lavg−H(s) in bits/pixel. Ideally, the redundancy of a good code should be zero.

METHODS AND STANDARDS FOR LOSSLESS COMPRESSION

	Digitized medical data

	Bitonal image transmission via a facsimile device also imposes such requirements.

Delay

Symbol-to-Codeword

Mapping

Probability

Model

Input Symbol

Codeword

Figure: A generic model for lossless compression

The combination of the probability modeling and the symbol-to-codeword mapping functions is usually referred to as entropy coding.

Message-to-Symbol Partitioning

As noted before, entropy coding is performed on a symbol by symbol basis.

Appropriate partitioning of the input messages into symbols is very important for efficient coding.

One could view one instance of a 256×256 multi-frame image as a single message, 2562=65536 long.

However, it is very difficult to provide probability models for such long symbols.

In practice, we typically view any image as a string of symbols drawn from the alphabet 0,1,2,…,255.

Differential Coding

If, say, the pixels in the image are in the order x1,x2,x3,…,xN, then instead of compressing these pixels, one might process the sequence of differentials yi=xi−xi−1, where i=1,2,…,N, and x0=0.

In compression terminology, yi is referred to as the prediction residual of xi.

0

# Preprocessing

0

255

255

-255

The Lossless JPEG Standard

	Differential coding to form prediction residuals.

	Residuals then coded with either a Huffman coder or an arithmetic coder

In lossless JPEG, one forms a prediction residual using "previous" pixels in the current line and/or the previous line.

The prediction residual for pixel X is r=y−X.

y=0

X

b

c

a

y=a

y=b

y=c

y=a+b−c

y=a+(b−c)2

y=b+(a−c)2

y=(a+b)2

The prediction residual

	Is computed modulo 216.

	Is expressed as a pair of symbols: the category and the actual value (magnitude).

The category represents the number of bits needed to encode the magnitude. This value is Huffman coded.

Example:	Magnitude 42 ⇒ Category 6

Residual 42 ⇒ (6, 6-bit code for 42).

Huffman code

If the residual is negative, then the code for the magnitude is the one's complement of its absolute value.

Codewords for negative residual always start wish a zero bit.

Example: Consider

X

b

c

a

a=100

b=191

c=100

X=180

y=(a+b)2

y=145

r=145−180=−35 ⇒ Category 6

Suppose Huffman code for six is 1110 then ⇒

−35 is coded by the 10-bit codeword 1110011100

Without entropy coding, −35 would require 16 bits.

| Category | Prediction Residual |
| --- | --- |
| 0 | 0 |
| 1 | -1, 1 |
| 2 | -3, -2, 2, 3 |
| 3 | -7, …, -4, 4, …, 7 |
| 4 | -15, …, -8, 8, …, 15 |
| 5 | -31, …,-16, 16, …, 31 |
| 6 | -63, …, -32, 32, …, 63 |
| 7 | -127, ..., -64, 64, …, 127 |
| 8 | -255, ..., -128, 128, ..., 255 |
| 9 | -511, ..., -256, 256, ..., 511 |
| 10 | -1023,..., -512, 512, ..., 1023 |
| 11 | -2047, ..., -1024, 1024, ..., 2047 |
| 12 | -4095, ..., -2048, 2048, ..., 4095 |
| 13 | -8191, ..., -4096, 4096, ..., 8191 |
| 14 | -16383, …,-8192, 8192, ..., 16383 |
| 15 | -32767, ..., -16384, 16384, ..., 32767 |
| 16 | 32768 |

STANDARDS FOR LOSSLESS COMPRESSION

Facsimile Compression Standards and Run-Length Coding Scheme

In every bitonal image there are large regions that are either all white or all black.

(position, value) ⇒ (run, value)

Such a mapping scheme is referred to as a run-length coding scheme.

Figure: Sample scanline of a bitonal image

The combination of a run-length coding scheme followed by a Huffman coder forms the basis of the image coding standards for facsimile applications. FACSIMILE COMPRESSION STANDARDS

	ITU-T Rec. T.4 (also known as Group 3).

1.	Modified Huffman (MH) code.

2.	Modified Read (MR) code.

	ITU-T Rec. T.6 (also known as Group 4).

Compression ratio:

	20:1 to 50:1 for business-type scanned documents.

	Severely degraded for images composed of natural scenes and rendered as bitonal images.

	JBIG