# SCALAR QUANTISATION

Let f represent a continuous scalar quantity which could be one of the following:

	pixel intensity

	transform coefficient

	image model parameter

	other

Suppose that only L levels are used to represent f.

This process is called amplitude quantisation.

Scalar quantisation

Each scalar is quantised independently.

Vector quantisation

Two or more scalars are quantised jointly, i.e., the vector formed by two or more scalars is quantised.

Let f denote an f that has been quantised.

f=Q(f)=ri, di−1<f≤di

Q:	quantisation operation

ri:	the L reconstruction levels, 1≤i≤L

di:	L+1 decision boundaries, 0≤i≤L

f can be expressed as

f=Q(f)=f+eQ, where eQ(f)=f−f

eQ:	quantisation noise

	(f−f)

	d(f−f)=f−f

	d(ffp−fp1p

The reconstruction and decision levels are often determined by minimizing some error criterion based on =…(f−f0)T(f−f0)pf(f0)df0.

Example:	Minimise the average distortion D, defined as follows:

D=E[d(f,f]=d(f0,f)pf(f0)df0

## Uniform quantisation

In uniform quantisation the reconstruction and decision levels are uniformly spaced.

D=E[eQTeQ]=E[(f−f)T(f−f)], 1≤i≤L and ri=di+di−12, 1≤i≤L

Figure 1.1:	Example of uniform quantisation. The number of reconstruction levels is 4, f is assumed to be between 0 and 1, and D is the result of quantising f. The reconstruction levels and decision boundaries are denoted by ri and di, respectively.

1/8

1/8

1

3/4

1/2

1/4

Figure 1.2:	Illustration of signal dependence of quantisation noise in uniform quantisation

Uniform quantisation may not be optimal!

Suppose f is much more likely to be in one particular region that in others.

It is reasonable to assign more reconstruction levels to that region!

Quantisation in which reconstruction and decision levels do not have even spacing is called non-uniform quantisation.

Optimum determination of ri and di depends on the error criterion used.

## Quantisation using the MMSE criterion

Suppose f is a random variable with a pdf pf(f0).

Using the minimum mean squared error (MMSE) criterion, we determine rk and dk by minimising the average distortion D given by

D=E[d(f,f)]=E[eQ2]=E[(f−f)2]pf(f0)(f−f0)2df0

Noting that f is one of the L reconstruction levels we write

D=i=1Lpf(f0)(ri−f0)2df0

To minimize D

∂D∂rk=0,		1≤k≤L

∂D∂dk=0,	1≤k≤L−1

d0=−∞

dL=∞

It is proven that from the above we get

rk=f0pf(f0)df0pf(f0)df0	1≤k≤L

dk=rk+rk+12			1≤k≤L−1

d0=−∞

dL=∞

Note that:

	The reconstruction level rk is the centroid of pf(f0) over the interval dk−1≤f0≤dk.

	The decision level dk except d0 and dL is the middle point between two reconstruction levels rk and rk+1.

	The above set of equations is a necessary set of equations for the optimal solution.

	For a certain class of pdf's including uniform, Gaussian, Laplacian, is also sufficient.

A quantiser based on the MMSE criterion is often referred to as Lloyd-Max quantiser.

0.9816

-0.9816

-1.5104

-0.4528

1.5104

0.4528

Figure 1.3:	Example of a Lloyd-Max quantiser. The number of reconstruction levels is 4, and the probability for f is Gaussian with mean 0 and variance 1.

2	VECTOR QUANTISATION

Let f=[f1,f2,…,fk]T denote an k-dimensional vector that consists of k real-valued, continuous-amplitude scalars fi.

f is mapped to another k-dimensional vector y=[y1,y2,…,yk]T

y is chosen from N possible reconstruction or quantisation levels

f=VQ(f)=yi,  f∈Ci

VQ is the vector quantisation operation

Ci is called the ith cell.

distortion measure:	d(f,f)=eQTeQ

quantisation noise:	eQ=f−f=VQ(f)−f

PROBLEM:	determine yi and boundaries of cells Ci

SOLUTION:	minimise some error criterion such as the average distortion measure D given by

D=E[d(f,f)]

D=E[eQTeQ]=E[(f−f)T(f−f)]=…(f−f0)T(f−f0)pf(f0)df0=i=1Nf0∈Ci…(ri−f0)T(ri−f0)pf(f0)df0

MAJOR ADVANTAGE OF VQ: performance improvement over scalar quantisation of a vector source.

That means

	VQ can lower the average distortion D with the number of reconstruction values held constant.

	VQ can reduce the required number of reconstruction values when D is held constant.



















Figure 2.1:	Example of vector quantisation. The number of scalars in the vector is 2, and the number of reconstruction levels is 9.